CSE220 · Bangladesh University of Engineering and Technology

Audio Signal Processing Toolkit

Team signal_pai_na

An interactive Python application that walks the complete audio DSP pipeline — sampling through reconstruction — on real uploaded audio or one-click synthetic demos, pairing every stage's math with a live, adjustable visualization.

Team Members

| Student ID | Name |
| --- | --- |
| 2305064 | Debo Protim Paul Turjjo |
| 2305072 | Md Rakibul Hasan Sumon |

01  A Brief Description

Most DSP coursework stays on paper: derive the DFT, sketch a Butterworth response, compute an SNR by hand. This project turns that same material into one running application — a mono, floating-point signal moves through sampling, framing, spectral analysis, filtering and reconstruction, and every stage is something you can drag a slider on and hear the result of, not just prove on a whiteboard.

The app ships three synthetic signal generators (a speech-like signal, a pure tone buried in noise with a known clean reference, and a song-like mixture with known foreground/background stems), so every technique has ground truth to be graded against even without a personal audio library. Real WAV/MP3 upload is fully supported alongside them.

02  Proposed Solution

Structure the app as one signal chain, one tab per stage, backed by DSP modules that know nothing about the UI — so the same functions that draw a plot in the browser can be imported and unit-tested on their own.

Input  →  Sampling  →  Time-Domain + Framing  →  FFT / STFT  →  Filter / Separate / Denoise  →  Reconstruction + Output

Every DSP module (audio_toolkit/*.py) is a plain NumPy/SciPy function with no Streamlit import — app.py is a thin UI layer of 10 tabs on top. This keeps the logic independently testable and reusable outside the browser, and keeps the app dependency-light enough to run anywhere without a GPU.

Tech stack:  Python 3 · NumPy · SciPy · librosa · SoundFile · Matplotlib · Streamlit

03  Relevant Concept(s) from Our Course

Ten concepts, ten tabs — each idea from lecture maps onto one working, adjustable panel in the app.

| Concept | How it's used | Module | Tab |
| --- | --- | --- | --- |
| Sampling Theorem — Nyquist–Shannon & Aliasing | Sample a tone below 2× its frequency and watch reconstruction fold into a lower “ghost” alias instead of the true tone. | sampling.py | Tab 3 |
| Time / Frequency Domain Duality | The same signal shown as a waveform and as a magnitude spectrum, side by side, updating together. | spectral.py | Tabs 2, 5 |
| Framing & Windowing | Splitting audio into overlapping 20–30 ms frames and applying a Hamming window before analysis, since audio isn't stationary over long spans. | framing.py | Tabs 4–6 |
| Discrete Fourier Transform (FFT) | Computing the magnitude spectrum of a whole signal or a single frame and reading off the dominant frequency. | spectral.py | Tab 5 |
| Short-Time Fourier Transform (STFT) | A spectrogram built from overlapping FFTs, exposing the FFT-size / hop-length trade-off between frequency and time resolution. | spectral.py | Tab 6 |
| Convolution & LTI Filtering | Butterworth low/high/band-pass/band-stop filters as second-order sections, applied via zero-phase filtfilt. | filters.py | Tab 7 |
| Signal Energy & Thresholding | Per-frame short-time energy combined with an FFT-derived speech-band ratio to classify Speech vs. Silence. | vad.py | Tab 4 |
| Spectral Subtraction | Modeling noisy = clean + noise in the frequency domain and subtracting an estimated noise magnitude, bin by bin. | noise_reduction.py | Tab 9 |
| Self-Similarity Spectral Masking | Nearest-neighbor STFT frames by cosine similarity separate repetitive (instrumental) content from non-repetitive (vocal) content — a REPET-SIM-style method. | separation.py | Tab 8 |
| Error Metrics — MSE, SNR & Correlation | Quantifying how close any two signals are — reconstructed vs. original, denoised vs. clean, resampled vs. source. | metrics.py | Tab 10 |

04  A Brief Ideation on the UI

Sketched as a persistent sidebar (file upload, three one-click demo buttons, and an audio player) plus one tab per pipeline stage — and built exactly that way, in Streamlit. Each tab pairs a short explanation of the math with a live control and a real Matplotlib visualization.

| 1 · File Info | Duration, sample rate, channel count, bit depth / estimated bitrate. |
| --- | --- |
| 2 · Waveform | Time-domain signal, zoomable to any sub-range. |
| 3 · Sampling & Aliasing | Adjustable tone frequency and sampling rate; reconstruction demo, MSE/SNR/correlation. |
| 4 · VAD | Frame-by-frame Speech/Silence classification via energy + speech-band ratio. |
| 5 · FFT Spectrum | Frequency content of the whole signal or a single selected frame. |
| 6 · Spectrogram | STFT magnitude over time; adjustable FFT size / hop length. |
| 7 · Filtering | Butterworth low/high/band/notch filter design with a live frequency-response plot. |
| 8 · Vocal/Instrumental Separation | Self-similarity spectral masking into foreground/background streams. |
| 9 · Noise Reduction | Spectral-subtraction denoising with adjustable oversubtraction/floor parameters. |
| 10 · Signal Comparison | Pick any two signals produced during the session and compute MSE, SNR, correlation. |

05  Running It

python3 -m venv .venv
source .venv/bin/activate
pip install -r requirements.txt
streamlit run app.py

Opens at localhost:8501. No audio file is required to start — the sidebar's three demo buttons generate speech-like, noisy-tone, and song-like signals (with ground truth) on the spot.